Requirements Documentation

[GitHub]: https://github.com/tableba/SmartHouse/tree/ali-iot/iot

Revision History	1

Requirements List	1

Requirements Descriptions	1

R1: Wokwi Device Simulation	2

R2: Wi-Fi and Backend Connection	2

R3: Device Registration	2

R4: Device Secret Authentication	2

R5: Device Heartbeat	2

R6: Device State Management	2

R7: Backend Command Handling	2

R8: Device State Reporting	3

R9: Multiple Device Support	3

SmartHouse – IoT

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 2026-07-27 | 1.0 | Created the initial requirements for the SmartHouse IoT subsystem based on the current project specification. | Ali Daoud |
| 2026-08-01 | 1.1 | Updated requirements progress. | Ali Daoud |
| 2026-08-14 | 1.2 | Updated progress after successful mock backend simulation | Ali Daoud |
| 2026-08-23 | 1.3 | Updated requirements after successful connection from Wokwi to the local backend through Cloudflare Tunnel. | Ali Daoud |
| 2026-08-30 | 1.4 | Finalized requirements after successful end-to-end IoT testing with Wokwi, backend, Firestore and Cloudflare Tunnel. | Ali Daoud |

Requirements List

| Requirement Name | Priority | Progress |
| --- | --- | --- |
| R1. Wokwi Device Simulation | Essential | 100% |
| R2. Wi-Fi and Backend Connection | Essential | 100% |
| R3. Device Registration | Essential | 100% |
| R4. Device Secret Authentication | Essential | 100% |
| R5. Device Heartbeat | Essential | 100% |
| R6. Device State Management | Essential | 100% |
| R7. Backend Command Handling | Essential | 100% |
| R8. Device State Reporting | Essential | 0% |
| R9. Multiple Device Support | Essential | 100% |

Requirements Descriptions

R1: Wokwi Device Simulation

The IoT subsystem shall use Wokwi and ESP32 to simulate smart-home devices without requiring physical hardware. The simulation shall include at least 10 logical devices using the device types supported by the backend.

R2: Wi-Fi and Backend Connection

The ESP32 simulation shall connect to Wokwi Wi-Fi and communicate with the SmartHouse backend through HTTP/HTTPS requests. During Wokwi development and testing, the locally running backend is exposed through a Cloudflare Tunnel. The backend base URL shall be configurable because the public Cloudflare Tunnel address may change when the tunnel is restarted.

Implementation status: Implemented and successfully tested. Wokwi connected to the locally running SmartHouse backend through a Cloudflare Tunnel, and real backend responses were received successfully.

R3: Device Registration

Each simulated device shall register with the backend by sending a unique ID, device name, valid device type, and initial state to the /api/devices/register endpoint. The backend shall return a device secret, which the ESP32 shall store for later authenticated communication.

Implementation status:
Implemented and successfully tested. Devices are registered from Wokwi through the Cloudflare Tunnel to the real SmartHouse backend. Each successfully registered device receives a backend-generated secret, which is stored by the ESP32 and used for later authenticated communication.

For repeated Wokwi demonstrations, the Firestore device collection is cleared before starting the simulation so that the devices can register again and receive new secrets.

R4: Device Secret Authentication

Each registered device shall use its unique device ID and secret when communicating with protected IoT endpoints. Requests with an invalid secret shall be rejected by the backend.

Implementation status: Secret-based authentication is implemented and successfully tested. The backend generates a unique secret during registration. The ESP32 stores the secret and uses it together with the device ID when sending authenticated heartbeat requests.

R5: Device Heartbeat

Each registered device shall send a heartbeat request to the /api/devices/heartbeat endpoint every 30 seconds. The request shall contain the device ID and secret so that the backend can determine whether the device is online or offline.

Implementation status: Implemented and successfully tested. Each registered device sends a heartbeat every 30 seconds using its device ID and secret. Successful heartbeat requests have been verified with HTTP 200 responses.

R6: Device State Management

Each simulated device shall maintain only the state fields relevant to its device type. Examples include power and brightness for lights, open status for doors and windows, speed for fans, temperature for temperature sensors, motion status for motion sensors, and active status for alarms.

R7: Backend Command Handling

The IoT subsystem shall retrieve desired device states from the backend and apply the received states to the corresponding simulated hardware.

Implementation status: Implemented and successfully tested against the real SmartHouse backend through the Cloudflare Tunnel. The ESP32 retrieves desired states through GET /api/devices/states every 3 seconds, matches the returned device IDs, and applies the states to the corresponding simulated Wokwi hardware. HTTP 200 responses from the real backend have been verified.

R8: Device State Reporting

The IoT subsystem shall report relevant sensor and device state changes to the backend.

Implementation status: Not implemented because no backend endpoint for reporting sensor/device states is currently available.

R9: Multiple Device Support

The IoT subsystem shall support at least 10 simulated smart-home devices. The devices may use the supported backend types: light, door, fan, coffee_machine, temperature_sensor, motion_sensor, alarm, and window. Each device shall have a unique ID, name, state, and authentication secret.