Test Plan Documentation

[GitHub]: https://github.com/tableba/SmartHouse/tree/ali-iot/iot

SmartHouse – IoT

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 2026-07-27 | 1.0 | Created the initial test plan for the SmartHouse IoT subsystem. | Ali Daoud |
| 2026-08-01 | 1.1 | Updated the test statuses after implementing and testing the Wokwi ESP32 simulation. | Ali Daoud |
| 2026-08-14 | 1.2 | Updated test results after mock backend simulation | Ali Daoud |
| 2026-08-22 | 1.3 | Added Wokwi simulation and Serial Monitor screenshots as test evidence. | Ali Daoud |

Revision History	1

Test Plan	2

Test Plan

Test Cases

| Requirement | Test | Status |
| --- | --- | --- |
| R1. Wokwi Device Simulation | Start the ESP32 simulation and verify that all configured devices initialize correctly. | Success |
| R2. Wi-Fi and Backend Connection | Connect the ESP32 to Wokwi Wi-Fi and verify that it can communicate with the backend URL. | Partially successful – Wi-Fi works. Real backend connection could not be tested because Wokwi Free does not support the required Private IoT Gateway. |
| R3. Device Registration | Register a device using a unique ID, valid name, type, and initial state. Verify that the backend returns a secret. | Success using mock backend – all 10 devices received simulated status 200 and mock secrets were stored. |
| R3. Device Registration | Attempt registration using an invalid device type and verify that the request is rejected. | Not tested – mock backend does not validate invalid device types. |
| R4. Device Secret Authentication | Send an authenticated request using the correct device ID and secret. | Success using mock backend – stored device secrets are used for heartbeat requests. |
| R4. Device Secret Authentication | Send a request using an invalid secret and verify that it is rejected. | Not tested – mock backend does not perform server-side secret validation. |
| R5. Device Heartbeat | Verify that each registered device sends a heartbeat every 30 seconds. | Success – all 10 devices send simulated heartbeats every 30 seconds. |
| R6. Device State Management | Verify that every device stores only state fields relevant to its device type. | Success |
| R7. Backend Command Handling | Retrieve simulated desired states and verify that the correct simulated devices change state. | Success using mock backend – desired states are retrieved every 3 seconds and applied to the Wokwi hardware. |
| R8. Device State Reporting | Change a device state and verify that the updated state is reported to the backend. | Blocked – backend endpoint for device state reporting is not available. |
| R9. Multiple Device Support | Run at least 10 devices with unique IDs and verify that registration and heartbeat communication work independently. | Success using mock backend – 10 logical devices register independently, store secrets, send heartbeats, and receive state updates. |

Figure 1. Wokwi simulation showing the ESP32 and connected SmartHouse devices.

Figure 2. Serial Monitor showing successful mock device registration and stored device secrets.